# Systems Analysis: Mathematical Frameworks and Algorithms

## Executive Summary

The Wi-Fi Optimizer Pro application leverages several branches of mathematics to model, simulate, and visualize electromagnetic wave propagation. The core systems rely on Computational Geometry for spatial reasoning, Physics Equations for signal attenuation, Optimization Algorithms for automated decision making, and Linear Algebra for 3D rendering.

---

## 1. Computational Geometry

The application treats the floor plan as a 2D Cartesian plane defined by vectors.

### 1.1 Line Segment Intersection (Ray Casting)

**Purpose**: To detect if a "wall" object obstructs the line of sight between two points (the Router and a Reciever).

**Implementation**: `MathUtils.getIntersection`

**Mathematical Concept**: Solving a system of linear equations derived from parametric lines.

Given two segments $P$ and $Q$:

P(t) = P_1 + t(P_2 - P_1)

Q(u) = Q_1 + u(Q_2 - Q_1)

The system solves for scalars $t$ and $u$. If $0 \le t, u \le 1$, the segments intersect. The solution uses Cramer's Rule (determinants) to solve the $2 \times 2$ matrix equation.

### 1.2 Point-to-Line Segment Distance

**Purpose**: Used in `grid.js` to detect which wall a user is clicking on.

**Implementation**: `FloorPlan.getWallAt`

**Mathematical Concept**: Vector Projection.

To find the closest point on a line segment $AB$ to a point $P$, we calculate the scalar projection $t$ of vector $\vec{AP}$ onto $\vec{AB}$:

t = \frac{\vec{AP} \cdot \vec{AB}}{\|\vec{AB}\|^2}

The value $t$ is clamped to $[0, 1]$ to ensure the closest point lies within the segment. The distance is then the Euclidean norm $\|\vec{P} - \text{ClosestPoint}\|$.

### 1.3 Euclidean Distance

**Purpose**: Fundamental metric for signal path loss.

**Formula**:

d = \sqrt{(x_2 - x_1)^2 + (y_2 - y_1)^2}

---

## 2. Physics of Signal Propagation

The simulation models electromagnetic wave behavior using the Log-Distance Path Loss Model, modified for indoor environments with obstacle attenuation.

### 2.1 The Free Space Path Loss (FSPL) & One-Slope Model

**Purpose**: To calculate the base signal degradation over distance.

**Formula**:

PL(d)_{dB} = L_0 + 10 \cdot n \cdot \log_{10}(d)

**$d$**: Distance in meters (converted from pixels via scale factor).

**$L_0$**: Path loss at reference distance (1m).

Implementation uses $L_0 = 40$ dB for 2.4GHz.

Implementation uses $L_0 = 47$ dB for 5.0GHz.

**$n$**: Path Loss Exponent. The code uses $n=2.0$ (Free Space logic), though typically indoor office environments might use $n=3.0$ to $3.5$.

**$\log_{10}$**: Converts the power ratio to decibels.

### 2.2 Obstacle Attenuation

**Purpose**: To account for signal absorption by walls.

**Formula**:

RSSI = P_{tx} - PL(d) - \sum_{i=1}^{k} W_i

Where $W_i$ is the attenuation factor of the $i$-th wall intersected by the ray.

**Concrete Wall**: 12 dB loss

**Door (Wood)**: 6 dB loss

**Window (Glass)**: 3 dB loss

---

## 3. Algorithmic Optimization

### 3.1 Heuristic Grid Search

**Purpose**: To automatically find the optimal router location.

**Algorithm Type**: Exhaustive Grid Search with Heuristic Evaluation.

**Process**:

**Discretization**: The floor plan is sampled at fixed intervals (e.g., $\Delta x = 40$ px).

**Evaluation**: For every candidate point $(x,y)$, a score is computed.

**Objective Function**:

S(x,y) = \sum_{p \in Grid} f(RSSI_p)

The heuristic step function $f$ (fitness) is:

$$f(r) = \begin{cases}

+10 & \text{if } r > -70 \text{ dBm} \\

+5 & \text{if } -80 < r \le -70 \text{ dBm} \\

-5 & \text{if } r \le -80 \text{ dBm}

\end{cases}$$

**ArgMax**: The algorithm returns $(x,y)$ that maximizes $S$.

---

## 4. 3D Rendering & Linear Algebra

The visualization uses Three.js which relies heavily on matrix mathematics.

### 4.1 Coordinate Transformations

**World Matrix**: Maps local object coordinates to world space.

**View Matrix**: Maps world space to camera space (inverse of Camera position/rotation).

**Projection Matrix**: Maps camera space to clip space (Perspective Projection).

The code explicitly handles 2D $\to$ 3D transformation:

FloorPlan $(x, y)$ $\to$ 3D World $(x, \text{height}, y)$ (Note: Three.js Y is 'Up', so 2D 'Y' becomes 3D 'Z').

### 4.2 Interpolation (Color Mapping)

**Purpose**: To render the heat map gradient.

**Math**: Linear Interpolation (Lerp) in HSL color space.

H = (1 - \text{normalized\_rssi}) \cdot 240^\circ

High Signal (1.0) $\to$ $0^\circ \cdot 240 + 0 = 0$ (Red? Logic in code treats 0 as bad, 240 as Blue/Good, or vice-versa. Code actually maps: Strong Signal $\to$ Green/Red, Weak $\to$ Blue). *Correction: Code implementation maps normalized \[0,1\] to hue \[240, 0\]. If 1 is best signal, Hue is 0 (Red). If 0 is worst, Hue is 240 (Blue).*

### 4.3 Ray Casting (3D)

**Purpose**: Mouse interaction with the 3D scene.

**Math**: Unprojecting a 2D vector from Screen Space $[-1, 1]$ into a 3D ray direction vector, then checking for triangle execution.

---

## 5. Numerical Methods

### 5.1 Discrete Field Approximation

The continuous electromagnetic field is impossible to calculate perfectly. The application uses Finite Difference principles by approximating the continuous field as a discrete Grid (Matrix) of values. The resolution (e.g., 10px) determines the trade-off between accuracy and computational cost ($O(N^2)$).